WINTER INTERNSHIP REPORT

Horizontal Subsurface Flow Constructed Wetlands for Domestic Wastewater Treatment: A Performance-Based Review

Submitted by

Mrityunjay Kumar

Department of Civil Engineering
National Institute of Technology Durgapur

Under the Guidance of

Dr. Mayur Shirish Jain

Assistant Professor

STWUTEE Research Group

Department of Civil Engineering

Indian Institute of Technology Indore

Simrol, Indore – 453552

Duration: 1 December 2025 – 30 December 2025

TABLE OF CONTENTS

Acknowledgement1

Abstract2

1.  Introduction2

2.  Methodology4

2.1  Search Strategy4

2.2  Inclusion & Exclusion Criteria4

2.3  Data Extraction and Comparative Evaluation Method4

3.  Fundamentals of HSSF-CW Systems5

3.1  Hydraulic Behaviour5

3.2  Physical, Chemical, and Biological Removal Processes5

3.3  Media Characteristics6

3.4  Role of Plants6

3.5  Key Limitations and Challenges7

4.  Global Development and Application of HSSF-CW8

5.  Historical Evolution of HSSF-CW9

6.  Research Gaps10

7.  Results10

7.1  Dissolved Oxygen (DO)10

7.2  pH11

7.3  Total Suspended Solids (TSS)11

7.4  Turbidity12

7.5  Volatile Suspended Solids (VSS)12

7.6  COD (Chemical Oxygen Demand)12

7.7  Nitrate (NO₃⁻) and Nitrite (NO₂⁻)13

7.8  Alkalinity14

7.9  Hardness14

7.10  Fluoride14

7.11  Chloride15

7.12  Biochemical Oxygen Demand (BOD)15

7.13  Phosphate16

7.14  ICP-OES Analysis16

8.  Design and Operational Considerations17

8.1  Hydraulic Retention Time (HRT)18

8.2  Hydraulic Loading Rate (HLR)18

8.3  Organic Loading Rate (OLR)18

8.4  Bed Depth, Porosity and Slope18

8.5  Media Characteristics18

8.6  Plant Species and Root Zone Characteristics19

8.7  Clogging Mechanism19

9.  Potential for Irrigation Reuse (Future Scope)19

10.  Comparison with Other Constructed Wetlands19

11.  Future Scope and Emerging Trends20

12.  Conclusion20

References21

Acknowledgement

I would like to express my sincere gratitude to Dr. Mayur Shirish Jain, Assistant Professor, Department of Civil Engineering, Indian Institute of Technology Indore, for providing me the opportunity to undertake this winter internship and for his valuable guidance throughout the course of this work.

I would also like to thank the STWUTEE Research Group, IIT Indore, for providing a supportive environment and necessary facilities to carry out this work.

I am highly thankful to Deborshee Sinha Sir for his constant guidance and support in the laboratory during the experimental work, and for mentoring me throughout the internship.

I would also like to thank Adarsh sir for his assistance in material procurement and other related work.

I express my gratitude to Shiksha ma’am for her help in conducting experiments, testing and presentation work.

I am thankful to my co-intern, Harshvardhan for his cooperation and support during the internship period.

I would also like to acknowledge Mr. Deepak, technician at IIT Indore workshop, for his assistance in fabrication and setup work.

I am grateful to the gardener at IIT Indore for arranging the Typha plant required for the study.

Finally, I would like to acknowledge the support of National Institute of Technology Durgapur in providing the academic foundation for this work.

Abstract

This study focuses on evaluating the performance of Horizontal Subsurface Flow Constructed Wetland (HSSF-CW) for treatment of domestic wastewater. The system was prepared using layers of sand and gravel along with Typha plant, and it was operated at a flow rate of 3.6 ml/min with a hydraulic retention time of 3.5 days.

The treatment performance was studied using different water quality parameters like BOD, COD, TSS, turbidity and some other chemical parameters. From the results, it was seen that there is good reduction in suspended solids, organic matter and turbidity, which shows that filtration and microbial activity are working properly in the system. The pH values remained mostly stable during the study. Nitrogen compounds showed some increase and decrease pattern, which indicates that nitrification and denitrification processes are taking place.

The study also shows that design factors like media type, flow rate and plants play an important role in treatment efficiency. The treated water shows some potential for reuse, but more testing is required before using it for irrigation.

Overall, HSSF constructed wetland is a simple, low-cost and useful method for domestic wastewater treatment, especially in areas where proper treatment systems are not available.

- Introduction

Global population expansion, rapid urbanisation and a shift in consumption pattern has dramatically increased the domestic wastewater volumes across the world. International estimates project a sharp rise in wastewater output over the coming years, straining treatment systems, especially in developing countries where infrastructure lags (UN Habitat and WHO, 2021). Centralised conventional sewage treatment plants (STPs), though effective for pollutant reduction, demand substantial upfront capital, expert operators, steady energy supply, and intricate maintenance routines that restrict their deployment in rural areas, small towns, and peri-urban zones (Parde et al., 2021).

These limitations have fuelled the nature-based solutions (NBS) as favourable practical wastewater management options. NBS emphasize the use of natural processes to enhance environmental resilience while reducing energy consumption and operational challenges (Tsatsou et al., 2023).  Among these many approaches, constructed wetlands (CWs) have emerged as the most reliable, low-cost, and environmentally friendly systems capable of treating domestic and municipal wastewater under a wide range of climatic and operational conditions (Chen et al., 2024).

Constructed wetlands are systems engineered to follow the physical, chemical, and biological pollutant removal processes of natural wetlands. Their advantages are very low energy consumption, simple operation, less sludge production and the ability to integrate wastewater treatment with ecological and landscape functions (Parde et al., 2021). Due to these advantages, CWs are being used for domestic wastewater treatment, stormwater management, and agricultural runoff control in both developed and developing countries (Chen et al., 2024).

Horizontal subsurface flow constructed wetland (HSSF-CW) is among the most frequently deployed types of constructed wetland systems around the globe. Untreated wastewater enters through one end and flows along gravel beds positioned beneath the surface to the other side, interacting with microbial communities, root systems, and filter media. This below-surface flow system largely reduces odour  and  human exposure to wastewater and risk of mosquito breeding (Ergaieg et al., 2021).

Pollutant removal in HSSF-CWs occurs by a combination of filtration, sedimentation, adsorption, microbial degradation, and the plant’s root action. Organic matter is mainly removed by microbial activities under aerobic conditions, while nutrients such as nitrogen and phosphorus are transformed the process of nitrification-denitrification, plant uptake, and media interactions (Mechanistic Understanding of the Pollutant Removal and Transformation Processes in the Constructed Wetland System, n.d.). The efficiency of these processes is influenced by design and operational parameters, including media characteristics, hydraulic retention time, loading rates, and vegetation type (Jamwal et al., 2021).

Alongside wastewater treatment, there is growing attention on reusing treated water for agriculture, mainly because freshwater resources are under increasing pressure. With farming consuming almost 70% of global freshwater, treated domestic wastewater offers a practical way to save water and recover nutrients (Christou et al., 2024). Reuse of treated effluent for irrigation can improve crop productivity; however, it requires compliance with water quality criteria related to salinity, nutrient concentrations, suspended solids, and pathogen levels to prevent adverse impacts on soil, crops, and human health (Ibrahim & Hammad, 2024).

Despite extensive research on the pollutant removal performance of HSSF-CWs, very few studies dedicately link treatment efficiency with irrigation reuse suitability. Most investigations focus primarily on conventional performance indicators such as BOD, COD, and nutrient removal, without explicitly assessing whether treated effluents meet international irrigation water quality standards (Mosquera-romero et al., 2023). Furthermore, the available literature offers only fragmented insights into how design parameters influence treatment performance alongside reuse suitability.

The present study aims to provide a systematic review of horizontal subsurface flow constructed wetlands used for domestic wastewater treatment, with a specific emphasis on their potential for agricultural irrigation reuse. By going through recent literature, this review evaluates treatment efficiencies, design and operational parameters, and effluent quality. The study further identifies key research gaps and scope for further studies.

- Methodology

To find literature on Horizontal Subsurface Flow Constructed Wetlands (HSSF-CW) for treating domestic wastewater for irrigational reuse purposes, a systematic literature review was conducted using the Scopus database.

The methodology for selecting and filtering articles was done using a structured search process with specific inclusion and exclusion criteria. The purpose of a systematic review is to focus on the topic and extract the needed information, and exclude the unwanted ones.

  - Search Strategy

The Scopus database was searched using different combinations of keywords related to the main topic of this review. The search mainly focused on terms associated with HSSF-CW systems, domestic wastewater, and irrigation reuse.

The keywords used for searching included “horizontal subsurface flow constructed wetland”, “HSSF constructed wetland”, “domestic wastewater”, “municipal wastewater”, “treatment efficiency”, “removal efficiency”, “effluent quality”, “wetland dimensions”, “media particle size”, “plant species”, “hydraulic retention time”, “hydraulic loading rate”, and “irrigation reuse”

Logical operators such as AND and OR were used to connect the keywords so that relevant studies could be identified more effectively. The search was carried out in the title, abstract, and keyword fields of Scopus to obtain the most relevant research articles.

2.2 Inclusion & exclusion criteria:

Only Scopus research papers published between 2015 and 2025 were considered for this study. Studies focusing on HSSF constructed wetlands treating domestic or municipal wastewater and reporting quantitative performance data were included. Articles written in English were selected.

Studies outside the mentioned time period, dealing with other wetland types, industrial wastewater, conference papers, editorials, and articles lacking measurable results were excluded.

2.3 Data extraction and comparative evaluation method

After the final selection of studies, relevant information was extracted from the papers, like wetland dimensions, type of plants used, media particle sizes, and the water quality parameters tested. The extracted data were then compared with different studies to evaluate the performance of HSSF-CW systems for domestic wastewater treatment and their compliance with irrigation water parameters.

- Fundamentals of HSSF-CW Systems

Horizontal Subsurface Flow Constructed Wetlands (HSSF-CWs) are human-engineered natural treatment systems in which wastewater flows horizontally below the surface through a saturated porous media bed, planted with wetland vegetation over it.(Vymazal, 2018) Constructed wetlands are green alternatives to STPs to improve the wastewater quality for discharge in natural streams or reuse for agricultural purposes.(Choi et al., 2012) These systems are widely applied for domestic wastewater treatment due to their lower energy demand, operational simplicity, low labour and maintenance costs and environmental sustainability. (Gholipour et al., 2020)

Figure 1: HSSF-CW setup at IIT Indore

3.1 Hydraulic Behaviour

In HSSF-CW systems, wastewater flows below the media surface under saturated conditions from the inlet to the outlet(Vymazal, 2018). The hydraulic behaviour is mainly controlled by media porosity, hydraulic conductivity, bed slope, and hydraulic retention time (HRT). Although plug-flow conditions are generally assumed in design, real systems often show hydraulic dispersion, dead zones, and short-circuiting, which can reduce treatment efficiency.(F. Zhao et al., 2024)

3.2 Physical, Chemical, and Biological Removal Processes

HSSF-CWs use the combined methods of physical, chemical, and biological means to eliminate pollutants. Through physical methods like sedimentation/screening, suspended solids can be removed directly from an HSSF-CW(Bôto et al., 2023). The chemical processes in a HSSF-CW, such as precipitation/adsorption, help reduce phosphorus and other metals, depending on which types of substrate/media used and plant species utilized to grow in a HSSF-CW (Wu et al., 2023). Organic matter removal occurs mainly through microbial breakdown in a treated CW and the breakdown of other contaminants are greatly influenced by the amount of dissolved oxygen available in a subsurface substrate/media (Sossalla et al., 2021).

3.3 Media characteristics

Selection of media is an important design aspect of the HSSF-CW system. It affects hydraulic performance and removal efficiency of pollutants (Mishra et al., 2024). Gravel(4-21mm) and sand (1-2mm) for supporting plants are most commonly used media due to their suitable permeability and surface area for good flow and growth of biofilm (Aiello et al., 2016). Particles of smaller sizes increase filtration and microbial activities but also increase the chance of clogging of system(Miranda et al., 2017). Coarser aggregates, in contrast, maintain higher hydraulic conductivity and increase oxygen presence but reduce the efficiency of filtration (F. Li et al., 2014). However, balanced particle sizes in the layer can improve the efficiency of the system (P. Zhao et al., 2025).

3.4 Role of plants

The role of plants in CWs is still not totally understood, but many studies and experiments show that CW with plantations shows better efficiency than unplanted ones.(Vymazal, 2011).

In constructed wetlands(CWs), plants contribute to removing pollutant by processes such as absorbing the nutrients or their degradation(Quan et al., 2016). They also do root-based filtration(Oon et al., 2017), increases oxygen in the root zone (B. Liu et al., 2018) And regulate pollutant retention time (Mustapha, 2018). Plants like Phragmites australis, Scirpus spp., Typha angustifolia, Glyceria maxima etc. are common wetland plants which are effective in pollutant absorption and can grow in hard conditions due to a strong root system (Stottmeister et al., 2003).

3.5 Key limitations and challenges

Despite having many advantages in Horizontal subsurface flow -CW, there are still some limitations to it, such as one major challenge is the clogging of the porous media. (Tatoulis et al., 2017 , Hickey et al., 2018). Clogging can occur due to various reasons, such as the deposition of suspended solids in wastewater. (Dai et al., 2025), plant root development and the growth of biofilm(Y. Li & Yuan, 2025), decrease of efficiency in colder regions due to reduced microbial activities at lower temperatures and plants' resilience to climate and quality of wastewater (S. Liu et al., 2024). Also, the requirement of land is there according to the quantity of water to be treated. Emission of Greenhouse gases (GHGs) is also a negative aspect of constructed wetlands. (Waters et al., n.d.).

- Global Development and Application of HSSF-CW

| Table 1. Performance and design characteristics of HSSF-CWs under different climatic conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Climatic condition | Country/site | Avg. temperature & rainfall | CW type | Dimensions/design parameters | Removal efficiencies (single column) | Plants used | Other relevant characteristics | Reference |
| Tropical semi-arid (southern India) | India – Berambadi village, Karnataka | Avg T: 22.9 °C; Rainfall: 915 mm yr⁻¹ (reported) | HSSF | 4.4 × 1.0 × 0.9 m; HRT = 3.7 d; HLR = 84 mm d⁻¹; porosity ≈ 32% | BOD₅: 67%; COD: –; TN: –; TP: –; TSS: – | Canna indica | Septic tank effluent polishing: planted vs unplanted comparison | (Jamwal et al., 2021) |
| Tropical semi-arid (southern India) | India – Berambadi village, Karnataka | Avg T: 22.9 °C; Rainfall: 915 mm yr⁻¹ | HSSF | 4.4 × 1.0 × 0.9 m; HRT = 3.7 d; HLR = 84 mm d⁻¹; porosity ≈ 32% | BOD₅: 61%; COD: –; TN: –; TP: higher than planted; TSS: higher than planted | – (unplanted) | Physico-chemical + microbial removal dominant | (Jamwal et al., 2021) |
| Humid subtropical (NE India) | India – Assam (IIT Guwahati, lab-scale) | Climate not reported; humid subtropical (country-based) | HSSF (baffled) | 0.95 × 0.32 × 0.55 m; HRT = 7 d; HLR = 0.031 m d⁻¹ | SO₄²⁻: 44–75%; pH: 2.1→6.4; Fe: 91.6%; Mn: low; Al: 59.7% | Typha latifolia | Organic substrate (cow manure + bamboo chips); AMD treatment | (Singh & Chakraborty, 2020) |
| Subtropical monsoon | India – Haridwar (Ganga basin) | Seasonal; winter & summer (reported qualitatively) | HSSF | Area = 79.17 m²; gravel depth = 0.75 m; HRT = 12–36 h | BOD: 55–94%; NO₃⁻-N: 55–91%; NH₄⁺-N: 55–91%; PO₄³⁻-P: 55–91% (season-dependent) | Typha latifolia, Phragmites australis, Colocasia esculenta | Seasonal performance; urban sewage polishing | (Rai et al., 2015) |
| Tropical humid | Ethiopia (Jimma University) | Mean T ≈ 22 °C; rainfall – | HSSF | 8 × 2 × 0.6 m; HLR ≈ 5 cm/d; HRT 4–7 d | BOD₅: 81%; COD: 65%; TSS: 76% | Cyperus papyrus | Irregular flow; shock-load tolerant | (Haddis et al., 2020) |
| Tropical humid | Ethiopia (Jimma University) | Mean T ≈ 22 °C; rainfall – | HSSF | 9 × 2 × 0.6 m; HLR ≈ 5 cm/d; HRT 4–7 d | BOD₅: 76%; COD: 62%; TSS: 75% | Scirpus validus | Compared with the unplanted control | (Haddis et al., 2020) |
| Tropical humid | Ethiopia (control bed) | Mean T ≈ 22 °C; rainfall – | HSSF | 10 × 2 × 0.6 m; HLR ≈ 5 cm/d; HRT 4–7 d | BOD₅: 48%; COD: 32%; TSS: 54% | – | Physico-chemical removal dominant | (Haddis et al., 2020) |
| Tropical humid | Uganda | Climate – (tropical, country-based) | HSSF | – | BOD₅: ~87%; N & P – | Cyperus papyrus | Pilot-scale system | (Haddis et al., 2020) |
| Tropical humid | Tanzania (Dar es Salaam) | Climate – (tropical, country-based) | HSSF | – | BOD₅: 71% (planted); 45% (unplanted) | Phragmites Mauritians | Polishing of pond effluent | (Haddis et al., 2020) |
| Tropical humid | Kenya | Climate – (tropical) | HSSF | – | COD: 59–65%; BOD – | Cyperus papyrus | Variable HLR | (Haddis et al., 2020) |
| Tropical & subtropical | Indonesia | Climate – (tropical) | HSSF | – | BOD: 70–90%; COD – | Cyperus alternifolius | Low-O&M domestic treatment | (Wijaya, 2016) |
| Tropical & subtropical | Indonesia | Climate – (tropical) | HSSF | – | Organic matter removal – | Phragmites spp. | Review of applied systems | (Wijaya, 2016) |
| Temperate | Poland | Mean T ≈ 7–9 °C | HSSF | ~3000 m²; shallow soil bed | N removal limited (denitrification dominant) | Phragmites australis | Low oxygen conditions | (Mucha et al., 2018) |
| Cold mountainous | Italy (Alps) | Cold mountainous climate | HSSF | Septic tank + HSSF bed | >60% removal for most pollutants | Phragmites australis | Space-limited sites | (Stentella et al., 2023) |
| Cold mountainous | Italy (Alps) | Cold mountainous climate | HSSF | – | >60% removal for most pollutants | Deschamps caespitose | Plant adaptation critical | (Stentella et al., 2023) |
| Cold region | Northern China | Winter T < 0 °C | HSSF | Insulated / geothermal | TN: 54.8%; NH₄⁺-N: 44.5%; TP: 77.7% | – | Geothermal anti-freezing | (Liang et al., 2020) |
| Cold region | China | Winter low T | HSSF | Heated/insulated | TN improvement ~120% vs control | Phragmites spp. | Compost-based heating | (Zhang et al., 2015) |

Table 1 indicates that HSSF constructed wetlands effectively remove pollutants in various climatic conditions. The efficiency of BOD removal across the systems ranges from 55% to 94%, demonstrating effective removal of organic matter in the majority of instances. In the semi-arid tropical regions of southern India, wetlands that were planted achieved better performance (67% BOD₅ removal) compared to those that were unplanted (61%). Tropical humid systems in Africa indicated BOD₅ removal rates of approximately 71% to 87%, while control beds demonstrated reduced efficacy. Under subtropical monsoon conditions in Haridwar, a reduction of organic matter and nutrients (NO₃⁻-N, NH₄⁺-N, and PO₄³⁻-P) between 55% and 91% was also noted, which shows the seasonal fluctuations. In humid subtropical NE India, addressing acid mine drainage demonstrated SO₄²⁻ elimination of 44–75% along with significant Fe removal (91.6%). Temperate and cold-climate systems exhibited decreased nutrient removal, especially during wintertime. Insulated or heated HSSF systems in cold areas showed better nutrient removal, achieving TN removal rates of 54.8% and TP rates of 77.7%

5. Historical evolution of HSSF-CW

The technology of the Constructed Wetland was first developed in the late 1960s. The initial examples of full-scale CWs with plants date back to 1967 in the Netherlands, Lelystad and Keszthely region, Hungary (Vymaza, 2022). Horizontal Subsurface Flow Constructed Wetlands (HSSF-CWs) have evolved as a reliable and green subsurface treatment technology because they allow wastewater to flow in saturated conditions and also under the surface, reducing odour problems and limiting human contact with sewage water. (Engida et al., 2020). Literature explains that horizontal subsurface flow constructed wetlands have developed into a reliable treatment unit as experience from field-scale systems improved understanding of their hydraulics, media Properties, and long-term operational performance. (Muduli et al., 2024). The early spread of HSSF-CWs was mainly driven by the need for decentralised and low-energy wastewater treatment solutions, particularly in regions where centralised treatment systems were difficult to implement or maintain (De Jesus et al., 2020). Initial HSSF-CW systems were largely designed to remove organic matter and suspended solids through passive hydraulic flow and microbial processes occurring within the subsurface media. (Wu et al., 2023). Operational challenges like clogging of the media and uneven flow distribution strongly influenced the historical development of HSSF-CWs and led to refinements in media selection, hydraulic loading control, and vegetation management practices. (Vymazal, 2018). Recent papers recognise HSSF-CWs as a mature treatment system that has progressed beyond basic wastewater treatment to address modern concerns such as nutrient management, emerging contaminants like acid mine discharge, and suitability for water reuse applications. (Tatoulis et al., 2017).

6. Research Gaps

Even though many studies are available on constructed wetlands, still some gaps are there. Long-term studies (more than 5 years) are very limited. Also, there is not much work connecting treated water quality with actual soil and crop impact.

Clogging behaviour under different climatic conditions is also not fully understood. Research on microplastics and other emerging pollutants is very less. Many studies also do not report proper design parameters, which makes comparison difficult.

There are also very few full-scale studies on irrigation reuse, especially in Indian conditions. So more practical and field-based studies are required in future.

7. Results
7.1. Dissolved Oxygen (DO)

The dissolved oxygen (DO) concentration shows an overall decreasing trend during the initial days of operation, followed by slight fluctuations towards the later stages. Initially, the DO value is relatively high (around 3.59 mg/L), which drops sharply to nearly 1.7 mg/L within the first day. This decline indicates rapid consumption of oxygen by microbial activity as organic matter starts getting degraded in the wetland system.

From day 2 to day 4, DO remains relatively stable (~1.7–1.8 mg/L), suggesting a balanced condition between oxygen consumption and limited oxygen transfer in the subsurface environment. A further decrease is observed around day 5 (0.8 mg/L), indicating more dominant anaerobic conditions, which are typical in HSSF constructed wetlands.

In the later days, a slight recovery in DO (up to ~1.6 mg/L) is observed, possibly due to plant root oxygen release and reduced organic load. Overall, the DO levels confirm that the system operates mainly under low-oxygen conditions, which supports processes like denitrification and organic matter degradation.

7.2 pH

The pH of the system remains relatively stable throughout the study, varying between approximately 6.4 and 7.0. Initially, the pH is around 6.79, followed by slight fluctuations with a minimum value near 6.4 and a maximum close to neutral (~7).

These variations may be due to ongoing biochemical processes such as organic matter degradation, nitrification, and denitrification. The slight decrease in pH during some days can be attributed to the formation of acidic intermediates, while the increase towards neutral values indicates buffering effects within the wetland media.

Overall, the pH remains within a favourable range for microbial activity, which is essential for efficient wastewater treatment. Additionally, the near-neutral pH makes the treated water suitable for irrigation reuse without causing adverse soil reactions.

7.3 Total Suspended Solids (TSS)

A significant reduction in Total Suspended Solids (TSS) is observed during the treatment process. The initial TSS concentration is very high (~1000 mg/L), which drops sharply to around 80–90 mg/L within the first day, indicating efficient removal of suspended particles.

This rapid reduction is mainly due to physical processes such as sedimentation and filtration through the porous media of the wetland. After the initial drop, TSS values remain relatively stable with minor fluctuations, indicating consistent system performance.

The overall removal efficiency of TSS is very high, demonstrating that HSSF constructed wetlands are highly effective in removing suspended solids. This also contributes to improved clarity and quality of the treated effluent.

7.4 Turbidity

The turbidity shows a sharp decrease from an initial high value (~14 NTU) to below 2 NTU within the first day. This indicates rapid removal of fine suspended particles.

After the initial reduction, turbidity values fluctuate slightly between 1.2 and 3.5 NTU but remain low throughout the treatment period. These variations may be due to minor disturbances or release of fine particles within the system.

7.5 Volatile Suspended Solids (VSS)

Volatile Suspended Solids (VSS), which represent the organic fraction of suspended matter, show a sharp decrease from around 960 mg/L to below 60 mg/L within the first day. This indicates rapid removal of biodegradable organic solids.

After the initial removal, VSS values remain relatively stable with slight variations, suggesting sustained microbial degradation within the wetland system.

The high reduction in VSS confirms that biological processes are actively contributing to the breakdown of organic matter.

7.6 COD (Chemical Oxygen Demand)

The COD values show an overall decreasing trend from an initial value of approximately 170 mg/L to around 90 mg/L towards the end of the study.

Although fluctuations are observed during intermediate days, the general trend indicates effective reduction of organic pollutants. The temporary increase around day 6–7 may be due to release of intermediate organic compounds or system disturbances.

Overall, the reduction in COD demonstrates the ability of the wetland system to remove both biodegradable and non-biodegradable organic matter, though not completely.

7.7 Nitrate (NO₃⁻) and Nitrite (NO₂⁻)

The variation of nitrate and nitrite concentrations over the study period shows a fluctuating trend, indicating dynamic nitrogen transformation processes within the HSSF constructed wetland system. As observed from the graphs (page 3) , nitrate levels show repeated rising and falling patterns, while nitrite appears intermittently in small concentrations.

Initially, nitrate concentration is relatively high (~3.6 mg/L), followed by a decrease in the early stage. In the intermediate phase, nitrate levels increase again, which suggests active nitrification where ammonia is converted into nitrite and subsequently into nitrate in localized aerobic zones, especially near plant roots. Correspondingly, small peaks of nitrite (~1 mg/L) are observed during this phase, indicating its role as an intermediate compound.

However, nitrite does not accumulate and quickly drops to near zero levels, showing that it is rapidly converted either into nitrate or further reduced. The repeated rise and fall of nitrate concentration throughout the study indicates the simultaneous occurrence of nitrification (increasing nitrate) and denitrification (decreasing nitrate) under varying oxygen conditions within different zones of the wetland.

Towards the final stage, a slight increase in nitrate is again observed, suggesting that nitrification continues to occur in certain regions of the system. Overall, the fluctuating behaviour of nitrate along with minimal nitrite accumulation indicates a balanced and ongoing nitrogen transformation process rather than complete removal.

7.8 Alkalinity

Alkalinity shows a decreasing trend from about 209 mg/L to around 90–100 mg/L. This reduction indicates consumption of alkalinity during biochemical processes such as nitrification.

The stabilization of alkalinity in later stages suggests buffering capacity within the system.

7.9 Hardness

Hardness decreases significantly from around 235 mg/L to nearly 70 mg/L over the treatment period. This reduction may be due to precipitation and adsorption of calcium and magnesium ions within the wetland media.

7.10 Fluoride

Fluoride concentration shows a decreasing trend from about 4 mg/L to nearly 1.1 mg/L. This indicates moderate removal, likely due to adsorption onto media.

7.11 Chloride

The chloride concentration shows a clear decreasing trend over the study period, which indicates that some removal is taking place in the HSSF constructed wetland system. As seen from the graph (page 4) , the value reduces from around 105 mg/L initially to nearly 50–55 mg/L towards the end.

The major reduction happens in the initial days, which may be due to dilution and interaction of dissolved salts with the wetland media. After that, only small fluctuations are observed, which can be due to changes in flow conditions or release of ions from the system.

Generally, chloride is not easily removed by biological processes, but in this case, the decrease suggests that physical and chemical mechanisms like adsorption or ion exchange are contributing to its reduction.

Overall, the system shows a moderate level of chloride removal, which is beneficial in reducing salinity and makes the treated water more suitable for irrigation use.

7.12 Biochemical Oxygen Demand (BOD)

The variation of Biochemical Oxygen Demand (BOD) over the study period shows a significant reduction, indicating effective removal of biodegradable organic matter in the HSSF constructed wetland system. Initially, the BOD value is high (~150 mg/L), which decreases sharply to around 100 mg/L within the first day. This rapid decline suggests intense microbial activity and quick degradation of easily biodegradable organic substances.

In the subsequent days, BOD values continue to decrease gradually, reaching around 70–80 mg/L. However, some fluctuations are observed during the intermediate phase, including slight increases around days 6 and 9. These variations may be attributed to the release of intermediate organic compounds, uneven flow conditions, or temporary changes in microbial activity within the system.

Towards the final stage, BOD reduces further to approximately 65 mg/L, indicating overall improvement in water quality. The continuous reduction in BOD confirms that biological processes play a dominant role in organic matter removal within the wetland.

Overall, the system demonstrates moderate to high efficiency in BOD removal, although complete stabilization is not achieved due to the complex and dynamic nature of treatment processes in constructed wetlands.

7.13 Phosphate

The phosphate concentration shows a strong decreasing trend throughout the treatment period, indicating effective phosphorus removal in the HSSF constructed wetland system. Initially, the phosphate value is around 9.7 mg/L, which decreases continuously with time and reaches nearly 0.2 mg/L towards the final stage.

A major reduction is observed during the first few days, suggesting rapid removal through adsorption and filtration processes within the wetland media. Plant uptake and microbial activities may also contribute to phosphate reduction. After day 5, the phosphate concentration becomes very low and remains almost stable with only minor fluctuations.

The overall high reduction in phosphate indicates that the wetland system is effective in controlling nutrient concentration in wastewater. Lower phosphate levels are beneficial because excess phosphorus can lead to eutrophication and algal growth in water bodies.

Overall, the HSSF constructed wetland shows very good performance in phosphate removal during the study period.

7.14 ICP-OES Analysis

The ICP-OES analysis was done to study the changes in different elements present in the wastewater during the treatment period. The values of boron (B), calcium (Ca), iron (Fe), potassium (K), magnesium (Mg) and sulphur (S) changed over different days, showing the effect of treatment processes inside the HSSF constructed wetland system.

Boron (B) concentration mostly decreased compared to the initial STP water. Lower boron concentration is useful because excess boron can affect plant growth if the treated water is used for irrigation.

Calcium (Ca) values showed some increase and decrease during the study. This may happen due to interaction of wastewater with sand and gravel media. Calcium is also related to water hardness.

Iron (Fe) concentration decreased during the treatment process. This reduction may be due to filtration and settling inside the wetland media. Lower iron concentration improves the quality of treated water.

Potassium (K) also showed an overall reduction, although some fluctuations were observed. Potassium is an important nutrient for plants, so moderate amounts in treated water may be beneficial for agricultural use.

Magnesium (Mg) concentration showed a decreasing trend during the study. Since magnesium contributes to hardness, reduction in magnesium indicates improvement in water quality.

Sulphur (S) values also changed during the treatment period, but overall reduction was observed compared to the initial water. This may be due to microbial activity and media interaction inside the system.

Overall, the ICP-OES results show that the HSSF constructed wetland system helps in improving the quality of wastewater by reducing different dissolved elements.

8. Design and Operational Considerations

Figure 2: Schematic Diagram of HSSF Constructed Wetland Setup

8.1 Hydraulic Retention Time (HRT)

The hydraulic retention time (HRT) of the system was maintained around 3.5 days. This is an important parameter because it decides how long the wastewater stays inside the wetland. If the water stays for more time, then better interaction happens with media, microbes and plant roots. In this setup, the HRT was sufficient, as we can see good reduction in BOD, COD and suspended solids. So overall, this HRT was suitable for proper treatment.

8.2 Hydraulic Loading Rate (HLR)

The system was operated at a flow rate of about 3.6 ml/min, which controls the hydraulic loading rate. A lower flow rate helps in avoiding short-circuiting and ensures that water flows properly through the media. In this case, the flow was kept steady, which helped in maintaining stable conditions and better pollutant removal.

8.3 Organic Loading Rate (OLR)

The organic loading rate depends on how much organic matter is entering the system. From the observed decrease in BOD and COD values, it can be said that the organic loading was within acceptable range. The system was not overloaded, so microorganisms were able to degrade organic matter effectively.

8.4 Bed Depth, Porosity and Slope

The wetland unit was constructed with dimensions of approximately 52 cm x 40 cm x 25 cm (length x width x height). The bed was made using different layers of sand and gravel, which provide both filtration and porosity. Proper depth is important so that enough contact time is available. The porous structure allows smooth flow and supports microbial growth. The system was designed for horizontal subsurface flow from inlet to outlet.

8.5 Media Characteristics

The media used in the system includes:

- Gravel (4–12 mm, around 15 cm thick)

- Gravel (1.2–20 mm, around 5 cm thick)

- Sand (1–4 mm, around 3 cm thick)

These materials were collected from the IIT Indore campus, and they were sieved to get proper size range. The combination of different sizes is important because larger particles help in maintaining flow, while smaller particles improve filtration and treatment efficiency.

8.6 Plant Species and Root Zone Characteristics

In this system, Typha plant was used, which was arranged with the help of the gardener at IIT Indore. Plants play an important role in constructed wetlands. They help in supplying oxygen to the root zone, support microbial activity and also take up some nutrients. The presence of Typha likely improved the overall performance of the system.

8.7 Clogging Mechanism

Clogging is one of the common problems in HSSF constructed wetlands. It can happen due to accumulation of suspended solids, growth of biofilm and plant roots. In this setup, proper size of media and controlled flow helped in reducing clogging. But still, in long-term operation, clogging may occur, so regular checking and maintenance is required.

9. Potential for Irrigation Reuse (Future Scope)

The treated wastewater from HSSF constructed wetland shows some potential for irrigation use, but proper evaluation is still needed. There are some international standards like WHO, FAO and USEPA which give limits for parameters like EC, TDS, nutrients and pathogens for safe irrigation. In this study, although many parameters improved, detailed comparison with these standards was not done.

Parameters like EC, TDS and nutrients are important because they affect soil and plant growth. Some nutrients like nitrogen can be useful for crops, but if they are in excess, they may damage soil quality. Also, presence of pathogens in treated water can create health risks if used directly.

Using treated wastewater can help in saving freshwater and can also improve soil fertility to some extent. But there are also concerns like salinity build-up and long-term soil impact. Safer irrigation methods like drip irrigation can be preferred instead of flood irrigation to reduce risks.

So overall, irrigation reuse is possible, but more practical testing is required before direct application.

10. Comparison with Other Constructed Wetlands

HSSF constructed wetlands are commonly used, but they can be compared with other types like vertical subsurface flow (VSSF) and hybrid systems.

HSSF systems mainly work under low oxygen conditions and are good for removal of organic matter and suspended solids. VSSF systems have better oxygen transfer and can improve nitrification. Hybrid systems combine both and give better overall performance.

In terms of cost, HSSF systems are simple and low maintenance, but they require more land area. Other systems may give better efficiency but are more complex. So selection of system depends on land availability, cost and treatment needs.

11. Future Scope and Emerging Trends

There are many new developments in constructed wetland systems. Use of improved media like biochar and industrial materials can increase treatment efficiency. Hybrid systems and aeration-based wetlands are also being studied.

Modern technologies like IoT monitoring, solar integration and machine learning are also being explored for better system control. Removal of emerging pollutants like microplastics is also becoming important.

In this study, some future work can be done. One important work is to perform seed germination test using treated wastewater on local crops of Madhya Pradesh like soybean, wheat and mustard. The germination rate and plant growth can be compared with tap water and irrigation canal water. This will help in checking whether treated water is suitable for agriculture or not.

Also, the system can be made in larger size to study real conditions. Instead of using STP inlet water from IIT Indore, wastewater can be taken directly from nearby areas like Simrol village. This will give more practical and real-life results.

12. Conclusion

In this study, the performance of Horizontal Subsurface Flow Constructed Wetland (HSSF-CW) for domestic wastewater treatment was analysed based on different water quality parameters. From the results, it is observed that the system is effective in reducing major pollutants like BOD, COD, TSS and turbidity. A significant reduction in suspended solids and organic matter was seen, especially in the initial days, which shows that physical filtration and microbial activity are playing an important role.

Parameters like pH remained stable throughout the study, which is suitable for treatment processes. Dissolved oxygen levels were low, indicating that the system mainly works under anaerobic conditions, which supports processes like denitrification. Nitrogen behaviour showed fluctuations, which indicates that both nitrification and denitrification processes are happening inside the system.

Other parameters like hardness, alkalinity, fluoride and chloride also showed reduction, although not completely. This shows that the system is capable of improving overall water quality, but complete removal of all contaminants is not possible due to the natural and dynamic nature of the wetland.

From the design point of view, parameters like HRT, flow rate and media composition played an important role in treatment efficiency. The use of layered media and Typha plant helped in improving the performance of the system.

Overall, it can be concluded that HSSF constructed wetlands are a simple, low-cost and effective method for domestic wastewater treatment. The treated water shows potential for reuse, but further studies like seed germination tests and comparison with irrigation standards are required before direct application.

References

Aiello, R., Bagarello, V., Barbagallo, S., Iovino, M., Marzo, A., & Toscano, A. (2016). Evaluation of clogging in full-scale subsurface flow constructed wetlands. Ecological Engineering, 95, 505–513. https://doi.org/10.1016/j.ecoleng.2016.06.113

Bôto, M. L., Dias, S. M., Crespo, R. D., Mucha, A. P., & Almeida, C. M. R. (2023). Removing chemical and biological pollutants from swine wastewater through constructed wetlands aiming reclaimed water reuse. Journal of Environmental Management, 326(August 2022). https://doi.org/10.1016/j.jenvman.2022.116642

Chen, G., Mo, Y., Gu, X., Jeppesen, E., Xie, T., Ning, Z., Li, Y., Li, D., Chen, C., Cui, B., & Wu, H. (2024). Sustainability of global small-scale constructed wetlands for multiple pollutant control. Npj Clean Water, 7(1), 1–13. https://doi.org/10.1038/s41545-024-00336-3

Choi, J. Y., Maniquiz, M. C., Geronimo, F. K., Lee, S. Y., Lee, B. S., & Kim, L. H. (2012). Development of a horizontal subsurface fl ow modular constructed wetland for urban runoff treatment. 000. https://doi.org/10.2166/wst.2012.306

Christou, A., Beretsou, V. G., Iakovides, I. C., Karaolia, P., Michael, C., Benmarhnia, T., Chefetz, B., Donner, E., Gawlik, B. M., Lee, Y., Lim, T. T., Lundy, L., Maffettone, R., Rizzo, L., Topp, E., & Fatta-kassinos, D. (2024). Sustainable wastewater reuse for agriculture. 5(July).

Dai, S., Wang, R., Lin, J., Zhang, G., Chen, Z., Li, L., & Shi, Q. (2025). Study on physical clogging process and practical application of horizontal subsurface flow constructed wetland. Scientific Reports, 15(1), 1–20. https://doi.org/10.1038/s41598-024-84159-9

De Jesus, F. L. F., De Matos, A. T., & De Matos, M. P. (2020). Efficiency of horizontal subsurface flow constructed wetlands cultivated with grasses of different root systems. Water Science and Technology: Water Supply, 20(8), 3318–3329. https://doi.org/10.2166/ws.2020.210

Engida, T., Wu, J. M., Xu, D., & Wu, Z. B. (2020). Review paper on horizontal subsurface flow constructed wetlands: Potential for their use in climate change mitigation and treatment of wastewater. Applied Ecology and Environmental Research, 18(1), 1051–1089. https://doi.org/10.15666/aeer/1801_10511089

Ergaieg, K., Msaddek, M. H., Kallel, A., & Trabelsi, I. (2021). Monitoring of horizontal subsurface flow constructed wetlands for tertiary treatment of municipal wastewater. Arabian Journal of Geosciences, 14(19). https://doi.org/10.1007/s12517-021-08419-y

Gholipour, A., Zahabi, H., & Stefanakis, A. I. (2020). Chemosphere A novel pilot and full-scale constructed wetland study for glass industry wastewater treatment. 247.

Haddis, A., Van der Bruggen, B., & Smets, I. (2020). Constructed wetlands as nature based solutions in removing organic pollutants from wastewater under irregular flow conditions in a tropical climate. Ecohydrology and Hydrobiology, 20(1), 38–47. https://doi.org/10.1016/j.ecohyd.2019.03.001

Hickey, A., Arnscheidt, J., Joyce, E., O’Toole, J., Galvin, G., O’ Callaghan, M., Conroy, K., Killian, D., Shryane, T., Hughes, F., Walsh, K., & Kavanagh, E. (2018). An assessment of the performance of municipal constructed wetlands in Ireland. Journal of Environmental Management, 210, 263–272. https://doi.org/10.1016/j.jenvman.2017.12.079

Ibrahim, L. A., & Hammad, D. M. (2024). Assessment of the impact of irrigating with treated wastewater on soil and crop trace metal accumulation. Water Science, 38(1), 324–336. https://doi.org/10.1080/23570008.2024.2357295

Jamwal, P., Raj, A. V., Raveendran, L., Shirin, S., Connelly, S., Yeluripati, J., Richards, S., Rao, L., Helliwell, R., & Tamburini, M. (2021). Evaluating the performance of horizontal sub-surface flow constructed wetlands: A case study from southern India. Ecological Engineering, 162(July 2020), 106170. https://doi.org/10.1016/j.ecoleng.2021.106170

Li, F., Lu, L., Zheng, X., & Zhang, X. (2014). Three-stage horizontal subsurface flow constructed wetlands for organics and nitrogen removal: Effect of aeration. Ecological Engineering, 68, 90–96. https://doi.org/10.1016/j.ecoleng.2014.03.025

Li, Y., & Yuan, L. (2025). Study on the long-term operation results of horizontal subsurface flow constructed wetlands for COD and nitrogen removal. Journal of Environmental Management, 393(May). https://doi.org/10.1016/j.jenvman.2025.127109

Liang, M. Y., Han, Y. C., Easa, S. M., Chu, P. P., Wang, Y. L., & Zhou, X. Y. (2020). New solution to build constructed wetland in cold climatic region. Science of the Total Environment, 719. https://doi.org/10.1016/j.scitotenv.2020.137124

Liu, B., Ji, M., & Zhai, H. (2018). Anodic potentials, electricity generation and bacterial community as affected by plant roots in sediment microbial fuel cell: Effects of anode locations. Chemosphere, 209, 739–747. https://doi.org/10.1016/j.chemosphere.2018.06.122

Liu, S., Zhang, Y., Feng, X., & Pyo, S. H. (2024). Current problems and countermeasures of constructed wetland for wastewater treatment: A review. Journal of Water Process Engineering, 57(November 2023). https://doi.org/10.1016/j.jwpe.2023.104569

Mechanistic understanding of the pollutant removal and transformation processes in the constructed wetland system. (n.d.). https://doi.org/10.1002/wer.1599

Miranda, S. T., Matos, A. T., Matos, M. P., Borges, A. C., & Baptestin, G. C. F. (2017). Characterization of clogging material from horizontal subsurface flow constructed wetland systems. Engenharia Agricola, 37(3), 463–470. https://doi.org/10.1590/1809-4430-eng.agric.v37n3p463-470/2017

Mishra, S. K., Dey Chowdhury, S., Bhunia, P., & Sarkar, A. (2024). Clogging in subsurface flow constructed wetlands: Mechanisms, influencing factors, measurements, modelling, and remediation. Ecological Engineering, 208(August). https://doi.org/10.1016/j.ecoleng.2024.107374

Mosquera-romero, S., Ntagia, E., Rousseau, D. P. L., Pr, A., & Esteve-nú, A. (2023). Environmental Science and Ecotechnology Water treatment and reclamation by implementing electrochemical systems with constructed wetlands. 16. https://doi.org/10.1016/j.ese.2023.100265

Mucha, Z., Wójcik, W., Jóźwiakowski, K., & Gajewska, M. (2018). Long-term operation of Kickuth-type constructed wetland applied to municipal wastewater treatment in temperate climate. Environmental Technology (United Kingdom), 39(9), 1133–1143. https://doi.org/10.1080/09593330.2017.1323014

Muduli, M., Choudharya, M., & Ray, S. (2024). A review on constructed wetlands for environmental and emerging contaminants removal from wastewater: traditional and recent developments. In Environment, Development and Sustainability (Vol. 26, Issue 12). Springer Netherlands. https://doi.org/10.1007/s10668-023-04190-0

Oon, Y. L., Ong, S. A., Ho, L. N., Wong, Y. S., Dahalan, F. A., Oon, Y. S., Lehl, H. K., Thung, W. E., & Nordin, N. (2017). Role of macrophyte and effect of supplementary aeration in up-flow constructed wetland-microbial fuel cell for simultaneous wastewater treatment and energy recovery. Bioresource Technology, 224, 265–275. https://doi.org/10.1016/j.biortech.2016.10.079

Parde, D., Patwa, A., Shukla, A., Vijay, R., Killedar, D. J., & Kumar, R. (2021). A review of constructed wetland on type, treatment and technology of wastewater. Environmental Technology and Innovation, 21(xxxx), 101261. https://doi.org/10.1016/j.eti.2020.101261

Quan, Q., Shen, B., Zhang, Q., & Ashraf, M. A. (2016). Research on phosphorus removal in artificial wetlands by plants and their photosynthesis. Brazilian Archives of Biology and Technology, 59(Specialissue), 1–9. https://doi.org/10.1590/1678-4324-2016160506

Rai, U. N., Upadhyay, A. K., Singh, N. K., Dwivedi, S., & Tripathi, R. D. (2015). Seasonal applicability of horizontal sub-surface flow constructed wetland for trace elements and nutrient removal from urban wastes to conserve Ganga River water quality at Haridwar, India. Ecological Engineering, 81, 115–122. https://doi.org/10.1016/j.ecoleng.2015.04.039

Singh, S., & Chakraborty, S. (2020). Performance of organic substrate amended constructed wetland treating acid mine drainage (AMD) of North-Eastern India. Journal of Hazardous Materials, 397(February). https://doi.org/10.1016/j.jhazmat.2020.122719

Sossalla, N. A., Nivala, J., Reemtsma, T., Schlichting, R., König, M., Forquet, N., van Afferden, M., Müller, R. A., & Escher, B. I. (2021). Removal of micropollutants and biological effects by conventional and intensified constructed wetlands treating municipal wastewater. Water Research, 201. https://doi.org/10.1016/j.watres.2021.117349

Stentella, R., Cislaghi, A., Rossi, L. M. W., Giupponi, L., Bona, E., Zambonardi, A., Rizzo, L., Esposto, F., & Bischetti, G. B. (2023). Ecological design of constructed wetlands in cold mountainous region: from literature to experience. Landscape and Ecological Engineering, 19(3), 401–415. https://doi.org/10.1007/s11355-023-00548-8

Stottmeister, U., Wießner, A., Kuschk, P., Kappelmeyer, U., Kästner, M., Bederski, O., Müller, R. A., & Moormann, H. (2003). Effects of plants and microorganisms in constructed wetlands for wastewater treatment. Biotechnology Advances, 22(1–2), 93–117. https://doi.org/10.1016/j.biotechadv.2003.08.010

Tatoulis, T., Akratos, C. S., Tekerlekopoulou, A. G., Vayenas, D. V., & Stefanakis, A. I. (2017). A novel horizontal subsurface flow constructed wetland: Reducing area requirements and clogging risk. Chemosphere, 186, 257–268. https://doi.org/10.1016/j.chemosphere.2017.07.151

Tsatsou, A., Frantzeskaki, N., & Malamis, S. (2023). Nature-based solutions for circular urban water systems: A scoping literature review and a proposal for urban design and planning. Journal of Cleaner Production, 394(May 2022), 136325. https://doi.org/10.1016/j.jclepro.2023.136325

UN Habitat and WHO. (2021). Progress on Wastewater Treatment – Global status and Acceleration Needs for SDG Indicator 6.3.1. In United Nations Human Settlements Program and World Health Organization (Vol. 1, Issue 1).

Vymaza, J. (2022). The Historical Development of Constructed Wetlands for Wastewater Treatment. Land, 11(2). https://doi.org/10.3390/land11020174

Vymazal, J. (2011). Plants used in constructed wetlands with horizontal subsurface flow: A review. Hydrobiologia, 674(1), 133–156. https://doi.org/10.1007/s10750-011-0738-9

Vymazal, J. (2018). Does clogging affect long-term removal of organics and suspended solids in gravel-based horizontal subsurface fl ow constructed wetlands ? 331(July 2017), 663–674.

Waters, A. A., Lizama-allende, K., Jaque, I., Ayala, J., Montes-atenas, G., & Leiva, E. (n.d.). Arsenic Removal Using Horizontal Subsurface Flow Constructed Wetlands : A Sustainable Alternative for. https://doi.org/10.3390/w10101447

Wijaya, D. S. (2016). A Review On Sub-Surface Flow Constructed Wetlands In Tropical And Sub-Tropical Countries. Open Science Journal, 1(2), 1–11. https://doi.org/10.23954/osj.v1i2.415

Wu, H., Wang, R., Yan, P., Wu, S., Chen, Z., Zhao, Y., Cheng, C., Hu, Z., Zhuang, L., Guo, Z., Xie, H., & Zhang, J. (2023). Constructed wetlands for pollution control. Nature Reviews Earth and Environment, 4(4), 218–234. https://doi.org/10.1038/s43017-023-00395-z

Zhang, L., Mu, L., Xiong, Y., Xi, B., Li, G., & Li, C. (2015). The development of a natural heating technology for constructed wetlands in cold climates. Ecological Engineering, 75, 51–60. https://doi.org/10.1016/j.ecoleng.2014.11.025

Zhao, F., Zhang, X., Xu, Z., Feng, C., Pan, W., Lu, L., & Luo, W. (2024). Review of hydraulic conditions optimization for constructed wetlands. Journal of Environmental Management, 370(August). https://doi.org/10.1016/j.jenvman.2024.122377

Zhao, P., Wang, R., Li, B., Zhang, G., Chen, Z., Wang, S., Hu, X., & Hao, R. (2025). Study on the optimization of hydrodynamic characteristics and pollutant removal efficiency in integrated vertical flow constructed wetlands. Scientific Reports, 15(1), 1–17. https://doi.org/10.1038/s41598-025-88785-9